/*!

\author J. Molina

\date 2013/01/17

\page page_rigid_body Rigid Body Dynamics

\see

- <a href="http://arxiv.org/abs/0811.2889"> Quaternions And Dynamics</a>, Basile Graf (2007)

- <a href="http://www.cs.cmu.edu/~baraff/pbm/rigid1.pdf"> An Introduction to Physically Based Modeling</a>, David Baraff (1997)

- Classical Dynamics, José and Saletan (1998) and/or Classical Mechanics, Goldstein et al. (2001)

- quaternion.h, rigid_body.h, and particle_solver.h

\section sec_design_rigid_body_quaternion Quaternions

Due to numerical stability, the use of Euler angles to represent rotations in simulations of rigid bodies is not recommended. One of the most popular alternatives is to use <a href="http://mathworld.wolfram.com/Quaternion.html"> quaternions </a> instead. A quaternion \f$\qtn{a}\f$ is defined as

\f[

\qtn{a} = a_0 + a_1\vec{i} + a_2\vec{j} + a_3\vec{k} =

(s, \vec{v})

\f]

where \f$s=a_0\f$ is the scalar (real) part and \f$\vec{v} = (a_1,a_2,a_3)\f$ is the vector (imaginary) part, \f$\vec{v}\in R^3\f$. The conjugate of the quaternion is then

\f[

\qtncnj{a} = (s, -\vec{v})

\f]

Addition of two quaternions \f$\qtn{a}=(s_a,\vec{v}_a)\f$ and \f$\qtn{b}=(s_b,\vec{v}_b)\f$ is simply

\f[

\qtn{a} + \qtn{b} = (s_a, \vec{v}_a) + (s_b, \vec{v}_b) =

(s_a + s_b, \vec{v}_a + \vec{v}_b)

\f]

and multiplication can be expressed as

\f[

\qtn{a}\circ\qtn{b} = (s_a s_b - \vec{v}_a\cdot\vec{v}_b,

s_a \vec{v}_b + s_b \vec{v}_a + \vec{v}_a \times \vec{v}_b)

\f]

Finally, the inverse of a quaternion is defined as

\f[

\qtn{a}^{-1} = \frac{\qtncnj{a}}{\norm{\qtn{a}}}

\f]

with the norm given by \f$\norm{\qtn{a}} = a_0^2 + a_1^2 + a_2^2 + a_3^2\f$.

\section sec_design_rigid_body_rotations Rotations

Assume we have a body coordinate system \f$\fancy{B}\f$, with origin at the center of mass, which is attached to the rotating rigid body. Let \f$x^\prime\f$ be the representation of a given vector \f$\vec{x}\f$ in \f$\fancy{B}\f$. Now let us take a second (non-inertial) reference system \f$\fancy{S}\f$, which also has as origin the center of mass of the body, and is at all times parallel to the (inertial) fixed laboratory frame \f$\fancy{L}\f$. The representation of the vector \f$\vec{x}\f$ in this fixed space system \f$\fancy{S}\f$ is denoted by \f$x\f$, and by definition it also gives the representation with respect to the lab frame \f$\fancy{L}\f$. If \f$R\f$ is the rotation matrix from the body frame to the space frame \f$\fancy{B}\rightarrow\fancy{S}\f$, the coordinates of the two representations are related through

\f{align*}{

x &= R x^\prime \qquad(\text{space frame}\;\fancy{S})\\

x^\prime &= R^{t} x\qquad(\text{body frame}\;\fancy{B})

\f}

The equation of motion for the orientation matrix is given by

\f{align*}{

\dot{R} &= \text{skew}\left(\vec{\omega}\right)\cdot R\qquad (\text{space frame}\;\fancy{S}) \\

\dot{R} &= R\cdot\text{skew}\left(\vec{\omega}^\prime\right)\qquad (\text{body frame}\;\fancy{B})

\f}

where

\f[

\text{skew}\left(\omega\right) = \begin{pmatrix}

0 & - \omega_3 & \omega_2 \\

\omega_3 & 0 & -\omega_1 \\

-\omega_2 & \omega_1 & 0

\end{pmatrix}

\f]

Using Rodrigues' rotation formula, the transformation between space and body reference frames can also be expressed as a rotation by an angle \f$\theta\f$ about a given (normal) vector \f$n\f$, as

\f[

x^\prime = x\cos{\theta} + n (x\cdot n)\left(1-\cos{\theta}\right)

+ \left(x\times n\right) \sin{\theta}

\f]

The inverse transformation being obtained by interchanging \f$x\f$ and \f$\theta\f$ with \f$x^\prime\f$ and \f$-\theta\f$, respectively.

In terms of quaternions, this takes the simple form

\f{align*}{

\qtn{x} &= (0,x) = \qtn{q}\circ\qtn{x}^\prime\circ\qtncnj{q}\qquad(\text{space frame}\;\fancy{S})\\

\qtn{x}^\prime &= (0,x^\prime) = \qtncnj{q}\circ\qtn{x}\circ\qtn{q}\qquad(\text{body frame}\;\fancy{B})

\f}

where the orientation quaternion (the equivalent of the rotation matrix) is defined as

\f[

\qtn{q} = \left(\cos{\frac{\theta}{2}}, \sin{\frac{\theta}{2}}\vec{n}\right)\qquad \norm{q}=1

\f]

and it obeys the following equations of motion

\f{align*}{

\dot{\qtn{q}} &= \frac{1}{2}\qtn{w}\circ\qtn{q}\qquad (\text{space frame}\;\fancy{S})\\

\dot{\qtn{q}} &= \frac{1}{2}\qtn{q}\circ\qtn{w}^\prime\qquad (\text{body frame}\;\fancy{B})

\f}

Finally, the rotation matrix can be expressed in terms of the quaternion components as

\f[

R = \begin{pmatrix}

q_0^2 + q_1^2 - q_2^2 - q_3^2 & 2(q_1q_2 - q_0q_3) & 2(q_1q_3 + q_0q_2)\\

2(q_1q_2 + q_0q_3) & q_0^2 - q_1^2 + q_2^2 - q_3^2 & 2(q_2q_3 - q_0q_1)\\

2(q_1q_3 - q_0q_2) & 2(q_2q_3 + q_0q_1) & q_0^2 - q_1^2 - q_2^2 + q_3^2

\end{pmatrix}

\f]

The elements of the diagonal, together with the normalization condition \f$\norm{q} = 1\f$, provide a set of four equations for four unknowns \f$q_i^2\f$, allowing one to easily obtain the components of the rotation quaternion from a rotation matrix.

Futhermore, in terms of the usual Euler angles (x-convention), the rotation matrix is given by

\f[

R = \begin{pmatrix}

\cos\psi\cos\phi - \cos\theta\sin\phi\sin\psi & -\sin\psi\cos\phi-\cos\theta\sin\phi\cos\psi & \sin\theta\sin\phi \\

\cos\psi\sin\phi + \cos\theta\cos\phi\sin\psi & -\sin\psi\sin\phi + \cos\theta\cos\phi\cos\psi &

-\sin\theta\cos\phi \\

\sin\theta\sin\psi & \sin\theta\cos\psi & \cos\theta

\end{pmatrix}

\f]

and simple inspection allows us to obtain the rotation angles from the rotation quaternions.

*/